|  | Elektrický senzor blízkosti Article number: 83.9900015 |
| --- | --- |

Attributes

| Description | Elektrický senzor blízkosti |
| --- | --- |
| Article number | 83.9900015 |
| Product key | SENSOR SME-8M-DS-24V-K-2,5-OE |
| Sensor type | REED contact |
| Contact type | NO (Normally Open) |
| Mounting interface | Insertable in T-slot, fastened by slotted clamping screw |
| Electrical connection | Cable, three-core |
| Cable length | 2,5 m |
| Material housing | Stainless steel / Nylon (Polyamide, PA) / Polyurethane (PUR) |
| Min./max. ambient temperature | -40 to 70 °C |
| Dust/water protection class | IP68 |
| Supply voltage | 24 V DC |
| Supply current | 0,08 A |
| Output voltage | 24 V DC |
| Output current | 80 mA |
| Length | 35 mm |
| Width | 5 mm |
| Thickness | 5 mm |
| Weight | 0,3 kg |