|  | Cleanflow magnet – poloautomatické čištění – statický - #200 mm Číslo výrobku: E0063099 |
| --- | --- |

Vlastnosti

| Popis | Cleanflow magnet – poloautomatické čištění – statický - #200 mm |
| --- | --- |
| Číslo výrobku | E0063099 |
| Produktový klíč | SECE-2020-02V-ES-F1M-B-B-BB-B- |
| Suitable for food production | For non-perishable foods only |
| Max. particle size | 10 mm |
| Min. Fe-particle size that can be caught | 0,03 mm |
| Production stop required during ferrous particles disposal | Yes |
| Cleaning (ferrous particles disposal) method | Semi-automatic (manual + pneumatic) |
| Ferrous particles disposal via | Manually sliding out magnets / cleaning = pneumatically |
| Ferrous particles disposal collection unit | No |
| Build-in length | 145 mm |
| Inlet shape | Square |
| Outlet shape | Square |
| Product inlet size | #200 mm |
| Product outlet size | #200 mm |
| Interface connection type/size | Square flange - 12xM8 |
| Max. capacity | 30 m³/h |
| Deflector grid | No |
| Materials in contact with product | AISI304 (SS 1.4301) / AISI316L (SS 1.4404) |
| Material seal rings between housing and magnetic bars | Silicone 714177 coated, 70° Shore, EC1935/FDA, red |
| Surface finishing (interior/exterior) | S1: Grit blasted, Ra 3 µm |
| Extra wear-resistant coating | No |
| Welds inside the product channel | WM: Continuously welded |
| Welds outside the product channel | WM: Continuously welded |
| Max. operating temperature | 60 °C |
| Min./max. ambient temperature | -10 to 60 °C |
| Max. operating pressure (pipeline pressure) | 2 bar |
| ATEX marking | Free of own ignition sources |
| Suited for use in ATEX zone | 20, 21 and 22 inside, 21 and 22 outside |
| Magnetic system | Magnetic bar in tube |
| Magnet quality | Neodymium N-52 |
| Magnetic bars position / behaviour during production | Static, fixed position 1 row |
| Magnetic bar size | Ø52,4 mm (wall thickness 1,2 mm) |
| Number of magnetic bars | 2 |
| Flux density on magnet tube in contact with product (±10%) | 12000 gauss |
| Pneumatic supply magnetic bars | 4 - 6 bar |
| Incl. single bar in/out sensoring | No |
| Incl. all bars magnet in/out sensoring | No |
| Safety device between housing and door/magnet unit | No |
| Length | 722 mm |
| Width | 438 mm |
| Height | 145 mm |
| Weight | 28 kg |