|  | Permanentní trubkový magnet - neodym - 10 m³/h Číslo výrobku: E0161038 |
| --- | --- |

Vlastnosti

| Popis | Permanentní trubkový magnet - neodym - 10 m³/h |
| --- | --- |
| Číslo výrobku | E0161038 |
| Produktový klíč | SPN-100-T1-01K-HT-F1m-S-T2-B |
| Tapered in-/outlet | Yes |
| Min. Fe-particle size that can be caught | 0,03 mm |
| Max. particle size | 16 mm |
| Production stop required during ferrous particles disposal | Yes |
| Cleaning (ferrous particles disposal) method | Manual |
| Ferrous particles disposal via | Manually by wiping |
| Build-in length | 750 mm |
| Interface connection type/size | Tube DN100 (Ø104x2 mm - acc. DIN 11850 R2/DIN 11866 A) |
| Max. capacity | 10 m³/h |
| Materials in contact with product | AISI316L (SS 1.4404) |
| Material seal between housing and door/magnet unit | Silicone VMQ, 70° Shore, EC1935/FDA, red |
| Surface treatment/finishing | S1: Grit blasted, Ra 3 µm |
| Extra wear-resistant coating | No |
| Welds inside the product channel | WM: Continuously welded |
| Welds outside the product channel | WM: Continuously welded |
| Max. operating temperature | 140 °C |
| Min./max. ambient temperature | -20 to 60 °C |
| Max. operating pressure (pipeline pressure) | 1 bar |
| ATEX marking | Free of own ignition sources |
| Suited for use in ATEX zone | 20, 21 and 22 inside, 21 and 22 outside |
| Magnetic system | Permanent Neoflux™ magnet |
| Magnet quality | Neodymium N-42SH |
| Magnet diameter | 65 mm |
| Magnet field strength (flux density) (±10%) | 11000 gauss |
| Safety device between housing and door/magnet unit | Sensor (Steute Ex HS Si 4 + Ex MC 4) (Ex II2GD) |
| Depth | 234 mm |
| Width | 261 mm |
| Height | 750 mm |
| Weight | 25 kg |