|  | Ovládací skříň - 3x400 V AC + PE; 50/60 Hz; 9 kW Číslo výrobku: ESBB055000 |
| --- | --- |

Vlastnosti

| Popis | Ovládací skříň - 3x400 V AC + PE; 50/60 Hz; 9 kW |
| --- | --- |
| Číslo výrobku | ESBB055000 |
| Produktový klíč | ROE-E-CB-NA-LC-HW-PT5K5 |
| Product name | Control box |
| Control box type | Wall mounted |
| Suitable for | Electro overband magnet |
| Surface treatment/finishing | - |
| Colour | Grey (Light grey): RAL7035 |
| Min./max. ambient temperature | -10 to 30 °C |
| Dust/water protection class | IP54 |
| EMC directive 2014/30/EU compliant | EMC_2014-30EU |
| EN 60204-1 compliant | Yes |
| IEC 61439-1 compliant | Yes |
| IEC 61439-2 compliant | Yes |
| Supply voltage | 3x400 V AC + PE; 50/60 Hz |
| Supply current | 75 A |
| Maximum fuse | 25 A |
| Output power | 5,5 kW |
| Output power to magnet | 6,3 kW |
| Output voltage to magnet | 118...124 V DC |
| Output current to magnet | 50 A |
| Output power to belt drive | 2,2 kW |
| Output voltage to belt drive | 400 V AC |
| Output current to belt drive | 5 A |
| Including rectifier | Yes |
| Including switch for local/remote control | Yes |
| Including safety switch | Yes, for transformer and motor |
| Main switch | Black/grey |
| Indicator light 'Running' (green) | Yes |
| Indicator light 'Alarm' (red) | Yes |
| Indicator light 'Standby' (white) | Yes |
| Emergency stop | Yes |
| Push button 'Reset emergency stop' | Yes |
| Push button ON | Yes |
| Push button OFF | Yes |
| Switch for local/remote control | Yes |
| Thermal protection | Yes |
| Depth | 300 mm |
| Width | 760 mm |
| Height | 760 mm |
| Weight | 110 kg |