|  | Neodymový závěsný separátor - 1200 mm Číslo výrobku: ROCC120380 |
| --- | --- |

Vlastnosti

| Popis | Neodymový závěsný separátor - 1200 mm |
| --- | --- |
| Číslo výrobku | ROCC120380 |
| Produktový klíč | ROP-C8-C-120-W-G-L-B-B-B-NA |
| Suitable for cross mounting | Yes |
| Suitable for inline mounting | Yes |
| Min. Fe-particle size that can be caught | 1 mm |
| Advised max. installation height | 550 mm |
| Ferrous particles disposal method | Automatic - continuous |
| Suitable for belt width | 1200 mm |
| Installation/transport interface | Lifting eye Ø40 mm (8x) |
| Material framework | Stainless steel |
| Material covering | Stainless steel |
| Surface treatment/finishing magnet(s) | Painted |
| Surface treatment/finishing rollers | Painted |
| Surface treatment/finishing frame | Grit blasted |
| Surface treatment/finishing covers | Grit blasted |
| Max. operating temperature | 60 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| Conveying length | 2070 mm |
| Belt width | 1200 mm |
| Belt speed | 1,5 m/s |
| Belt type | Rubber, with cleats (EP500/3 6+2) |
| Belt cleat type | T100/10 |
| No. of cleats | 12 |
| Magnetic system | Permanent Neoflux® magnet |
| Magnet quality | Neodymium N-38 |
| Magnet dimensions | 1460x1175x411 mm |
| Main pole dimensions | 1230x1175x411 mm |
| Motor drive | SEW electro motor with gear box |
| Motor power | 5,5 kW |
| Motor frequency | 50 Hz |
| Motor voltage | 230 (Δ) / 400 (Y) V AC |
| Motor position / operating side | Left (seen in transport direction) |
| Motor rotational speed (out) | 79 rpm |
| Motor dust/water protection class | IP54 |
| Duty cycle | 100%, 24 hours/day |
| Including central greasing point on both sides for bearings | No |
| Length | 3000 mm |
| Width | 1860 mm |
| Height | 830 mm |
| Weight | 3750 kg |