|  | Cleanflow™ magnet – ruční čištění – rotační - #300 mm Číslo výrobku: SCRB3030021 |
| --- | --- |

Vlastnosti

| Popis | Cleanflow™ magnet – ruční čištění – rotační - #300 mm |
| --- | --- |
| Číslo výrobku | SCRB3030021 |
| Produktový klíč | SECR-B-3030-12E-N-NA-F5m-B-B |
| Suitable for food production | Yes |
| Max. particle size | 10 mm |
| Min. Fe-particle size that can be caught | 0,03 mm |
| Production stop required during ferrous particles disposal | Yes |
| Cleaning (ferrous particles disposal) method | Manual |
| Ferrous particles disposal via | Manually taking out magnets + pulling them out of extractor |
| Build-in length | 300 mm |
| Inlet shape | Square |
| Outlet shape | Square |
| Product inlet size | #300 mm |
| Product outlet size | #300 mm |
| Interface connection type/size | Square flange - 20xM8 |
| Electrical connection | To connection box |
| Max. capacity | 18 m³/h |
| Version housing | Welded |
| Number of magnetic rotors | 1 |
| Materials in contact with product | AISI316 (SS 1.4401) / AISI316L (SS 1.4404) |
| Material seal between housing and door/magnet unit | Silicone VMQ, 40° Shore, EC1935/FDA, red |
| Motor oil: food grade | Yes |
| Surface finishing (interior/exterior) | S5: Vibratory polished, Ra 0,4 µm, ex. welds and end faces |
| Extra wear-resistant coating | No |
| Welds inside the product channel | Wm: Continuously welded |
| Welds outside the product channel | Wm: Continuously welded where possible |
| Max. operating temperature | 60 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| ATEX marking | - |
| Magnetic system | Magnetic bar in extractor tube |
| Magnet quality | Neodymium N-42 |
| Magnetic bars position / behaviour during production | Rotating, magnetic bars placed in a rotor |
| Magnetic bar size | Extractor tube Ø25 mm; magnetic bar Ø23 mm |
| Number of magnetic bars | 12 |
| Flux density on (internal) magnetic bar (±10%) | 10700 gauss |
| Flux density on magnet tube in contact with product (±10%) | 8000 gauss |
| Motor drive | SEW electro motor with gear box |
| Motor power | 0,37 kW |
| Motor current | 1,82 (Δ) / 1,05 (Y) A |
| Motor frequency | 50/60 Hz |
| Motor voltage | 230 (Δ) / 400 (Y) V AC |
| Motor rotational speed (out) | 36 rpm |
| Motor dust/water protection class | IP65 |
| Incl. double seal with air purging on shaft (max. 0.1 bar) | Yes |
| Incl. all bars magnet in/out sensoring | No |
| Safety device between housing and door/magnet unit | Switch (key type, Steute AZ16-zvk, Ex II 2GD) |
| Potential-free contact for ‘door closed’ signal | 250 V AC max., 230 V DC max. |
| Potential-free contact for ‘motor on’ signal | 600 V AC max., 250 V DC max. |
| Length | 557 mm |
| Width | 550 mm |
| Height | 300 mm |
| Weight | 54 kg |