|  | Elektromagnetický závěsný blok - 900 mm Číslo výrobku: SEBM037001 |
| --- | --- |

Vlastnosti

| Popis | Elektromagnetický závěsný blok - 900 mm |
| --- | --- |
| Číslo výrobku | SEBM037001 |
| Produktový klíč | RBE-EP7-080x090-090-A-NA |
| Min. Fe-particle size that can be caught | 3 mm |
| Advised max. installation height | 210-320 mm |
| Production stop required during ferrous particles disposal | Yes |
| Cleaning (ferrous particles disposal) method | Automatic |
| Ferrous particles disposal via | By switching off electromagnet |
| Suitable for belt width | 900 mm |
| Installation/transport interface | Lifting eye Ø40 mm (4x) |
| Material housing | Steel |
| Surface treatment/finishing | Painted |
| Colour housing | Red (Traffic red): RAL3020 |
| Max. operating temperature | 80 °C |
| Min./max. ambient temperature | -10 to 40 °C |
| Dust/water protection class | IP54 |
| Magnetic system | Electromagnet with air cooling |
| Magnet power | 3700 W |
| Magnet current | 34 A |
| Magnet voltage | 110 V DC |
| Duty cycle | 100%, 24 hours/day |
| Associated control unit (not included) | ESBB904001 |
| Length | 1000 mm |
| Width | 810 mm |
| Height | 428 mm |
| Weight | 1200 kg |