|  | Kaskádový magnet - 140 m³/h Číslo výrobku: SRKP080035 |
| --- | --- |

Vlastnosti

| Popis | Kaskádový magnet - 140 m³/h |
| --- | --- |
| Číslo výrobku | SRKP080035 |
| Produktový klíč | SEK-P-0800-3-FN-DC-Ex |
| Max. particle size | 10 mm |
| Min. Fe-particle size that can be caught | 0,5 mm |
| Production stop required during ferrous particles disposal | Yes |
| Cleaning (ferrous particles disposal) method | Automatic |
| Ferrous particles disposal via | Discharge chute by pneumatically actuated valve |
| Build-in length | 885 mm |
| In-/outlet shape | Rectangular |
| Product inlet size | 800x200 mm |
| Product outlet size | 800x200 mm |
| Ferrous particles outlet size | 800x140 mm |
| Interface connection type/size | Rectangular flanges with hole pattern acc. to drawing |
| Max. capacity | 140 m³/h |
| Materials in contact with product | AISI304 (SS 1.4301) |
| Surface finishing (interior/exterior) | Painted |
| Welds inside the product channel | WB: Welded shut |
| Welds outside the product channel | WB: Welded shut |
| Colour magnet(s) | Red (Traffic red): RAL3020 |
| Colour housing | Grey (Platinum grey): RAL7036 |
| Colour inspection hatch | Grey (Platinum grey): RAL7036 |
| Colour safety guards | Yellow (Zinc yellow): RAL1018 |
| Max. operating temperature | 100 °C |
| Min./max. ambient temperature | -5 to 40 °C |
| ATEX marking | Ex II 1/2D c T130 °C Da/Db |
| Suited for use in ATEX zone | 20, 21 and 22 inside, 21 and 22 outside |
| Magnetic system | Permanent plate magnet - extra strong |
| Magnet quality lowest stage | Neodymium N-38 |
| Magnet quality other stages | Anisotropic ferrite GSFD-33 |
| Magnet dimensions | 790x202x83 mm |
| Number of magnets | 3 |
| Magnet field strength (flux density) (±10%) | 2600 gauss |
| Solenoid valve electric supply | 24 V DC |
| Pneumatic supply | Max. 6 bar |
| Control | Signal (minimal 15 sec) from control room |
| Including integrated valve box for Fe-particles disposal | Yes |
| Incl. cleaning sequence control unit | Yes |
| Incl. wear plate (first step) | Yes |
| Depth | 1012 mm |
| Width | 1036 mm |
| Height | 885 mm |
| Weight | 316 kg |