|  | Einbaumagnettrommel - 600 mm Article number: E0061188 |
| --- | --- |

Attributes

| Description | Einbaumagnettrommel - 600 mm |
| --- | --- |
| Article number | E0061188 |
| Product key | RMD-06-32-RS-B-OR-30S2 |
| Max. particle size | 100 mm |
| Min. Fe-particle size that can be caught | 0,5 mm |
| Ferrous particles disposal method | Automatic - continuous |
| Magnetic field depth | 100 mm |
| Working width | 600 mm |
| Interface/connection | Shaft Ø35mm for gear motor (right-hand) |
| Max. capacity | 134 m³/h |
| Shell thickness | 3 mm |
| Material shell | Stainless steel |
| Max. operating temperature | 60 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| No. of cleats | 2 |
| Magnetic system | Permanent Neoflux® magnet - axial magnetic field |
| Magnet quality | Neodymium (NdFeB) |
| Flux density on shell (±10%) | 3000 gauss |
| Diameter | 320 mm |
| Length | 1043 mm |
| Width | 380 mm |
| Height | 380 mm |
| Weight | 114 kg |