|  | Magswitch Greifer Artikel Nummer: MS-8140320 |
| --- | --- |

Eigenschaften

| Beschreibung | Magswitch Greifer |
| --- | --- |
| Artikel Nummer | MS-8140320 |
| Produktschlüssel | HMS-AR70 HDC-70MM |
| Type | AR |
| Switching | Pneumatic |
| Dimension lifting surface | 92x72 mm |
| Advised working load | 1425 N |
| Remark Advised working load | Under ideal conditions - with safety factor 3 acc. EN13155 |
| Max. magnetic tear off force | 4270 N |
| Remark max. magnetic tear off force | Under ideal conditions |
| Min. advised sheet thickness | 12 mm |
| Mounting interface | Threaded hole M8 (10x) |
| Pneumatic connection | G1/8" (2x) |
| Colour | Yellow / Black / - (not painted) |
| Min./max. ambient temperature | -20 to 80 °C |
| Operating pressure | 2 - 10 bar |
| Magnetic system | Permanent Neoflux® magnet |
| Magnetic layout (number of magnet poles) | 12 |
| Magnet quality | Neodymium (NdFeB) |
| Incl. replaceable pole shoes (for alt. product shapes) | - |
| Length | 110 mm |
| Width | 110 mm |
| Height | 210 mm |
| Weight | 6 kg |