|  | Außenpolmagnet – Manuelle Schnellreinigung - 50 m³/h Artikel Nummer: SBPF180001 |
| --- | --- |

Eigenschaften

| Beschreibung | Außenpolmagnet – Manuelle Schnellreinigung - 50 m³/h |
| --- | --- |
| Artikel Nummer | SBPF180001 |
| Produktschlüssel | SBP-1818-F-F1B-B-B |
| Max. particle size | 50 mm |
| Min. Fe-particle size that can be caught | 1 mm |
| Production stop required during ferrous particles disposal | Yes |
| Cleaning (ferrous particles disposal) method | Manual |
| Ferrous particles disposal via | Manually by moving away the extractor plate |
| Build-in length | 400 mm |
| In-/outlet shape | Square |
| Product inlet size | #180 mm |
| Product outlet size | #180 mm |
| Interface connection type/size | Square flanges with hole pattern acc. to drawing |
| Max. capacity | 50 m³/h |
| Deflector | Removable |
| Materials in contact with product | AISI304 (SS 1.4301) |
| Material seal between housing and door/magnet unit | Silicone sponge rubber VMQ, white |
| Surface finishing (interior/exterior) | S1: Grit blasted, Ra 3 µm |
| Surface treatment/finishing magnet(s) | Painted |
| Extra wear-resistant coating | No |
| Welds inside the product channel | WB: Welded shut |
| Welds outside the product channel | WB: Welded shut |
| Colour magnet(s) | Blue (Ultramarine blue): RAL 5002 |
| Colour housing | - (not painted) |
| Max. operating temperature | 100 °C |
| Min./max. ambient temperature | -20 to 60 °C |
| Magnetic system | Permanent ferrite magnet |
| Magnet quality | Anisotropic ferrite F-33 |
| Magnet dimensions | 310x180x92 mm |
| Number of magnets | 2 |
| Flux density on extractor plate (±10%) | 2800 gauss |
| Safety device between housing and door/magnet unit | No |
| Depth | 355 mm |
| Width | 551 mm |
| Height | 400 mm |
| Weight | 85 kg |