|  | Elektro-Aushebemagnet - 800 mm Artikel Nummer: SEBM030001 |
| --- | --- |

Eigenschaften

| Beschreibung | Elektro-Aushebemagnet - 800 mm |
| --- | --- |
| Artikel Nummer | SEBM030001 |
| Produktschlüssel | RBE-EP6-070x080-080-A-NA |
| Min. Fe-particle size that can be caught | 3 mm |
| Advised max. installation height | 180-280 mm |
| Production stop required during ferrous particles disposal | Yes |
| Cleaning (ferrous particles disposal) method | Automatic |
| Ferrous particles disposal via | By switching off electromagnet |
| Suitable for belt width | 800 mm |
| Installation/transport interface | Lifting eye Ø40 mm (4x) |
| Material housing | Steel |
| Surface treatment/finishing | Painted |
| Colour housing | Blue (Ultramarine blue): RAL 5002 |
| Max. operating temperature | 80 °C |
| Min./max. ambient temperature | -10 to 40 °C |
| Dust/water protection class | IP54 |
| Magnetic system | Electromagnet with air cooling |
| Magnet power | 3000 W |
| Magnet current | 25 A |
| Magnet voltage | 120 V DC |
| Duty cycle | 100%, 24 hours/day |
| Associated control unit (not included) | ESBB903001 |
| Length | 898 mm |
| Width | 710 mm |
| Height | 438 mm |
| Weight | 970 kg |