|  | Magnettrommel im Gehäuse - Basistyp - 215 m³/h Artikel Nummer: SRTK120054 |
| --- | --- |

Eigenschaften

| Beschreibung | Magnettrommel im Gehäuse - Basistyp - 215 m³/h |
| --- | --- |
| Artikel Nummer | SRTK120054 |
| Produktschlüssel | SETK-125-SD-B-30S2-NA-FZB-B-B |
| Max. particle size | 100 mm |
| Min. Fe-particle size that can be caught | 0,5 mm |
| Production stop required during ferrous particles disposal | No |
| Cleaning (ferrous particles disposal) method | Automatic |
| Ferrous particles disposal via | Discharge chute |
| In-/outlet shape | Rectangular |
| Product inlet size | 1200x250 mm |
| Product outlet size | 1340x275 mm |
| Dust extraction opening size | 1200x75 mm |
| Ferrous particles outlet size | 1340x150 mm |
| Max. capacity | 215 m³/h |
| Rotational speed | 21 rpm |
| Surface speed | 33 m/min |
| Shell thickness | 3 mm |
| Materials in contact with product | Steel / rubber (NBR) |
| Surface treatment/finishing | Painted |
| Welds inside the product channel | WB: Welded shut |
| Welds outside the product channel | WB: Welded shut |
| Colour housing | White (Traffic white): RAL 9016 |
| Colour side panels / inspection hatch | Blue (Ultramarine blue): RAL 5002 |
| Max. operating temperature | 60 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| Magnet width | 1200 mm |
| No. of cleats | 2 |
| Magnetic system | Permanent ferrite magnet with flux boost |
| Magnet quality | Anisotropic ferrite F-33 + Neodymium N-38 |
| Magnet diameter | 506 mm |
| Flux density on shell (±10%) | 3000 gauss |
| Motor drive | SEW electro motor with gear box |
| Motor power | 0,75 kW |
| Motor frequency | 50 Hz |
| Motor voltage | 230 (Δ) / 400 (Y) V AC |
| Motor dust/water protection class | IP54 |
| Depth | 835 mm |
| Width | 1977 mm |
| Height | 775 mm |
| Weight | 816 kg |