|  | Magnetfilter - Industriell - Halbautomatische Reinigung - DN150 Artikel Nummer: SSFNP15038 |
| --- | --- |

Eigenschaften

| Beschreibung | Magnetfilter - Industriell - Halbautomatische Reinigung - DN150 |
| --- | --- |
| Artikel Nummer | SSFNP15038 |
| Produktschlüssel | SFN-150-IF-05Q-HT-F1M-S-B-B |
| Suitable for food production | For non-perishable foods only |
| Max. particle size | 10 mm |
| Min. Fe-particle size that can be caught | 0,03 mm |
| Production stop required during ferrous particles disposal | Yes |
| Cleaning (ferrous particles disposal) method | Semi-automatic (manual + pneumatic) |
| Ferrous particles disposal via | Manually sliding out magnets / cleaning = pneumatically |
| Build-in length | 460 mm |
| In-/outlet shape | Round |
| Interface connection type/size | DN150 acc. EN 1092-1 PN16 |
| Pneumatic connection | For 6 mm hose |
| Pressure drop (for water, at 1m/s) | 5 - 40 mbar |
| Magnet unit guidance for easy cleaning | Yes |
| Materials in contact with product | AISI316 (SS 1.4401) / AISI316L (SS 1.4404) |
| Material seal between housing and door/magnet unit | Silicone VMQ, 75° Shore, EC1935/FDA, red |
| Surface finishing (interior/exterior) | S1: Grit blasted, Ra 3 µm |
| Welds inside the product channel | Wm: Continuously welded |
| Welds outside the product channel | Wm: Continuously welded where possible |
| Max. operating temperature | 140 °C |
| Min./max. ambient temperature | -10 to 40 °C |
| Max. operating pressure (pipeline pressure) | 10 bar |
| Test pressure | 15 bar |
| Magnetic system | Magnetic bar in tube |
| Magnet quality | Neodymium N-42SH |
| Magnetic bars position / behaviour during production | Static, fixed position |
| Magnetic bar size | Ø34 mm |
| Number of magnetic bars | 5 |
| Flux density on magnet tube in contact with product (±10%) | 9400 gauss |
| Pneumatic supply magnetic bars | 4 (recommended) - 6 (max.) bar |
| Length | 724 mm |
| Width | 622 mm |
| Height | 460 mm |
| Weight | 94 kg |