|  | Pneumatischer Magnetgreifer - 40 mm Artikel Nummer: TPGC040078 |
| --- | --- |

Eigenschaften

| Beschreibung | Pneumatischer Magnetgreifer - 40 mm |
| --- | --- |
| Artikel Nummer | TPGC040078 |
| Produktschlüssel | HGR-SQ-040-FL-R-M-S |
| Switching | Pneumatic: compressed air or vacuum |
| Dimension lifting surface | Ø40 mm |
| Advised working load | 77 N |
| Remark Advised working load | Under ideal conditions - with safety factor 3 acc. EN13155 |
| Max. magnetic tear off force | 230 N |
| Remark max. magnetic tear off force | Under ideal conditions |
| Min. advised sheet thickness | 2 mm |
| Mounting interface | Threaded hole M5 (4x) + Threaded hole M8 |
| Pneumatic connection | M5 (2x) |
| Material housing | Aluminium / AISI316L (SS 1.4404) (screw cap) |
| Surface treatment/finishing | Anodized |
| Colour | Red |
| Min./max. ambient temperature | 0 to 70 °C |
| Switching vacuum pressure | -0,3 bar |
| Operating pressure | Max.: 6, recommended: 4 bar |
| Air consumption per cycle (1x up, 1x down) | 0,16 l |
| Magnetic system | Permanent Neoflux™ magnet |
| Magnetic layout (number of magnet poles) | 4 |
| Magnet quality | Neodymium N-45 |
| Incl. friction pad on magnetic surface | No |
| Length | 40 mm |
| Width | 40 mm |
| Height | 63 mm |
| Weight | 0,22 kg |