|  | Magnet unit - 12xN42-Ø30 mm - SECR3030 Article number: E0047666 |
| --- | --- |

Attributes

| Description | Magnet unit - 12xN42-Ø30 mm - SECR3030 |
| --- | --- |
| Article number | E0047666 |
| Product key | SECR-S-3030 MAGN PLATE 12B NdF |
| Product name | Magnet plate with magnetic bars for Rotating cleanflow |
| Suitable for food production | For non-perishable foods only |
| Materials in contact with product | AISI316 (SS 1.4401) / AISI316L (SS 1.4404) |
| Material seal between housing and door/magnet unit | Silicone VMQ, 40° Shore, EC1935/FDA, red |
| Motor oil: food grade | Yes |
| Surface finishing (interior/exterior) | S5: Vibratory polished, Ra 0,4 µm, ex. welds and end faces |
| Welds inside the product channel | Wm: Continuously welded |
| Welds outside the product channel | Wm: Continuously welded where possible |
| Magnet quality | Neodymium N-42 |
| Magnetic bar size | Magnetic bar Ø23 mm |
| Number of magnetic bars | 12 |
| Flux density on (internal) magnetic bar (±10%) | 10700 gauss |
| Flux density on magnet tube in contact with product (±10%) | 8000 gauss |
| Length | 295 mm |
| Diameter | Ø205 mm |
| Weight | 12 kg |