|  | Control box - 3x400 V AC + PE; 50 Hz Article number: E0131082 |
| --- | --- |

Attributes

| Description | Control box - 3x400 V AC + PE; 50 Hz |
| --- | --- |
| Article number | E0131082 |
| Product key | REC-E-CB-NA-PLI-PI-FD2 |
| Product name | Control box |
| Control box type | Floor mounted |
| Suitable for | Eddy current separator |
| Material enclosure and door | Steel |
| Surface treatment/finishing | S6: Powder coated (C4) |
| Colour | Grey (Light grey): RAL7035 |
| Min./max. ambient temperature | 0 to 40 °C |
| Dust/water protection class | IP54 |
| EMC directive 2014/30/EU compliant | EMC_2014-30EU |
| EN 60204-1 compliant | Yes |
| IEC 61439-1 compliant | Yes |
| IEC 61439-2 compliant | Yes |
| Supply voltage | 3x400 V AC + PE; 50 Hz |
| Supply current | 40 A |
| Maximum fuse | 40 A |
| Output power | 22,1 kW |
| Output power to belt drive | 1,1 - 2,2 kW |
| Output voltage to belt drive | 400 V AC |
| Output current to belt drive | 4,7 A |
| Output power to vibr. feeder | 2x1,0-1,2-1,7 kW |
| Output voltage to vibr. feeder | 400 V AC |
| Output current to vibr. feeder | Max. 3,2 A |
| Output power to magnetic roller | 5,5-7,5-11,0 kW |
| Output voltage to magnetic roller | 400 V AC |
| Output current to magnetic roller | 21,7 A |
| Output power to magnetic drum separator | 0,55-1,1 kW |
| Output voltage to magnetic drum separator | 400 V AC |
| Output current to magnetic drum separator | Max. 2,6 A |
| Incl. pot. free contacts for PLC | Yes |
| Including MCC - I/O contacts | Yes |
| Including plug connection between control box and cable set | Yes |
| Including profinet connection | Yes |
| Including IOT connection, LAN/WI-FI | Yes |
| Main switch | Red/yellow |
| Indicator light 'Running' (green) | Yes |
| Indicator light 'Alarm' (red) | Yes |
| Push button 'Reset emergency stop' | Yes |
| Touchscreen for operating and setting the machine | Yes |
| Thermal protection | Yes |
| Rotation detection belt | Yes |
| Depth | 460 mm |
| Width | 1200 mm |
| Height | 1300 mm |
| Weight | 160 kg |