|  | Timing belt permanent magnetic conveyor Article number: E0157855 |
| --- | --- |

Attributes

| Description | Timing belt permanent magnetic conveyor |
| --- | --- |
| Article number | E0157855 |
| Product key | HCT-F-B100-2500-C-N-N |
| Max. sheet thickness | 5 mm |
| Interface/connection | Shaft Ø35mm for gear motor (right-hand) |
| Min./max. ambient temperature | -5 to 50 °C |
| Conveying length | 2500 mm |
| Belt width | 100 mm |
| Max. belt speed | 3 m/s |
| Max. acceleration | 6 m/s² |
| Belt type | BATK10-PAZ |
| Magnetic system | Permanent ferrite magnet |
| Magnet quality | Anisotropic ferrite F-33 |
| Magnet power | 47 W |
| Magnet voltage | 24 V DC |
| Motor drive | Not included |
| Length | 1610 mm |
| Width | 240 mm |
| Height | 110 mm |
| Weight | 35 kg |