|  | Lightweight magnetic palletizer - 1000x1000 mm Article number: E0168571 |
| --- | --- |

Attributes

| Description | Lightweight magnetic palletizer - 1000x1000 mm |
| --- | --- |
| Article number | E0168571 |
| Product key | HPA-N-I5-AL-1050-1050-S3 |
| Pallet size | 1000 x 1000 mm |
| Suspension unit type | Robot arm conn. PCD200; 8xØ13,5 + 4xØ12H7, PCD160; 4xØ10H7mm |
| Pneumatic connection | For 8 mm hose |
| Electrical connection | To sensor connection box |
| Material housing | Aluminium |
| Surface treatment/finishing | Painted |
| Colour painted parts | Red (Traffic red): RAL3020 |
| Min./max. ambient temperature | -20 to 40 °C |
| Operating pressure | 6 - 8 bar |
| Air consumption | 8 liter/stroke (with 2 cylinders, 6 bar operating pressure) |
| Magnetic system | Permanent Neoflux™ magnet - medium pole - low magnets |
| Magnet quality | Neodymium N-35 |
| Magnet dimensions | 1022 x 1022 mm |
| Power supply sensor connection box | 12-30 V DC; max. 10 W |
| Incl. pallet grippers | No |
| Incl. vacuum grippers | No |
| On/off sensors with terminal box | Yes |
| Depth | 1050 mm |
| Width | 1050 mm |
| Height | 390 mm |
| Weight | 190 kg |