|  | Magnetic palletizer for gantry crane - 1200x800 mm Article number: E0168591 |
| --- | --- |

Attributes

| Description | Magnetic palletizer for gantry crane - 1200x800 mm |
| --- | --- |
| Article number | E0168591 |
| Product key | HPA-N-IM-SS-1250-850-S7 |
| Pallet size | 1200 x 800 mm |
| Suspension unit type | Shock absorbing anchor points M16 (4x) |
| Pneumatic connection | For 8 mm hose |
| Electrical connection | To sensor connection box |
| Material housing | Stainless steel |
| Surface treatment/finishing | S1: Grit blasted, Ra 3 µm |
| Min./max. ambient temperature | -20 to 40 °C |
| Operating pressure | 6 - 8 bar |
| Air consumption | 8 liter/stroke (with 2 cylinders, 6 bar operating pressure) l |
| Magnetic system | Permanent Neoflux® magnet - medium pole |
| Magnet quality | Neodymium N-35 |
| Magnet dimensions | 1222 x 822 mm |
| Power supply sensor connection box | 12-30 V DC; max. 10 W |
| Control | Electrical |
| Incl. pallet grippers | No |
| Incl. vacuum grippers | No |
| On/off sensors with terminal box | Yes |
| Depth | 1250 mm |
| Width | 850 mm |
| Height | 390 mm |
| Weight | 225 kg |