|  | Electronic magnetic gripper - 90 mm Article number: EHSQ090000 |
| --- | --- |

Attributes

| Description | Electronic magnetic gripper - 90 mm |
| --- | --- |
| Article number | EHSQ090000 |
| Product key | HGE-SQ-090-FR-EP-P-I |
| Switching / power supply | Electrical, with integrated electronic control |
| Dimension lifting surface | Ø96 mm |
| Advised working load | 750 N |
| Advised working load with friction pad | 720 |
| Advised working load without friction pad | 750 N |
| Remark Advised working load | Under ideal conditions - with safety factor 3 acc. EN13155 |
| Max. magnetic tear off force | 2250 N |
| Max. magnetic tear off force with friction pad | 2160 N |
| Max. magnetic tear off force without friction pad | 2250 N |
| Remark max. magnetic tear off force | Under ideal conditions |
| Max. shear force with friction pad | 540 N |
| Max. shear force without friction pad | 360 N |
| Min. advised material thickness (for optimal magnet force) | 10 mm |
| Mounting interface | Threaded hole M6 (8x) + Threaded hole M8 (4x) |
| Electrical connection | Single male connector 2 + 8 pins |
| Withstands acceleration forces up to | 20 m/s² |
| Material friction pad/ring | NBR 70° Shore A |
| Material housing | Aluminium / Steel |
| Surface treatment/finishing | Anodized / nickel plated |
| Colour | Red |
| Min./max. ambient temperature | 5 to 40 °C |
| Dust/water protection class | IP53 |
| Magnetic system | Electropermanent, reversible AlNiCo magnet & NdFeB magnets |
| Nominal supply voltage | 24 V DC |
| Peak current draw during switching time | 7 A @ 24 V DC |
| Rated current draw logical control | 0,2 A |
| Duty cycle | 12x 'ON' & 12x 'OFF'/ min |
| Incl. removable friction pad on magnetic surface | Yes |
| Thermal protection | Yes |
| Inductive sensor for product detection | Yes |
| Length | 106 mm |
| Width | 96 mm |
| Height | 125 mm |
| Weight | 4,1 kg |