|  | Door safety switch - NC/NO - Ex2D Article number: EITA042006 |
| --- | --- |

Attributes

| Description | Door safety switch - NC/NO - Ex2D |
| --- | --- |
| Article number | EITA042006 |
| Product key | CODED MAGN SWITCH XCS Ex 10M T |
| Sensor type | Coded magnetic safety switch |
| Contact type | 1 NC/1 NO |
| Mounting interface | Mounting hole Ø3,5 mm (2x) |
| Electrical connection | Pre-wired cable (4x0,25 mm²) |
| Cable length | 10 m |
| Material housing | Thermoplastic |
| Min./max. ambient temperature | -25 to 85 °C |
| ATEX marking | Ex II 2D Ex tb IIIC T135 °C Db IP67 |
| Suited for use in ATEX zone | 21 and 22 |
| Dust/water protection class | IP66 |
| EN 60079-0 compliant | Yes |
| EN 60079-11 compliant | Yes |
| EN 60079-31 compliant | Yes |
| EN ISO 13849-1 compliant | Yes |
| Supply voltage | 24 V DC |
| Supply current | 0,1 A |
| Output voltage | 24 V DC |
| Output current | 100 mA |
| Length | 51 mm |
| Width | 16 mm |
| Thickness | 7 mm |
| Weight | 0,1 kg |