|  | Magswitch gripper AR110 Article number: MS-8140740 |
| --- | --- |

Attributes

| Description | Magswitch gripper AR110 |
| --- | --- |
| Article number | MS-8140740 |
| Product key | HMS-AR110 HDC-110MM |
| Type | AR |
| Switching | Pneumatic |
| Dimension lifting surface | 150x114 mm |
| Advised working load | 4800 N |
| Remark Advised working load | Under ideal conditions - with safety factor 3 acc. EN13155 |
| Max. magnetic tear off force | 14500 N |
| Remark max. magnetic tear off force | Under ideal conditions |
| Min. advised sheet thickness | 35 mm |
| Mounting interface | Threaded hole M10 (8x) |
| Pneumatic connection | G1/8" (2x) |
| Colour | Yellow / Black / - (not painted) |
| Min./max. ambient temperature | -20 to 80 °C |
| Operating pressure | 4 - 10 bar |
| Magnetic system | Permanent Neoflux™ magnet |
| Magnetic layout (number of magnet poles) | 12 |
| Magnet quality | Neodymium (NdFeB) |
| Incl. replaceable pole shoes (for alt. product shapes) | - |
| Length | 190 mm |
| Width | 165 mm |
| Height | 430 mm |
| Weight | 25 kg |