|  | Add-on magnetic head pulley - 1000 mm Article number: RHAH15100003 |
| --- | --- |

Attributes

| Description | Add-on magnetic head pulley - 1000 mm |
| --- | --- |
| Article number | RHAH15100003 |
| Product key | RHP-H153R18-100-RSB-BA |
| Shaft type | Non-replaceable |
| Max. particle size | 40 mm |
| Min. Fe-particle size that can be caught | 0,03 mm |
| Min. weakly magnetic particles that can be caught | 0,1 mm |
| Ferrous particles disposal method | Automatic - continuous |
| Magnetic field depth | 10 mm |
| Suitable for belt width | 1000 mm |
| Roller width | 1222 mm |
| Interface/connection | Shaft Ø40h9 (2x) |
| Max. operating temperature | 60 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| Magnet width | 992 mm |
| Magnetic system | Permanent Neoflux® magnet |
| Magnetic layout | Radial pole |
| Magnet quality | Neodymium N-42 |
| Magnet field strength (flux density) (±10%) | 9000 gauss |
| Motor drive | Not included |
| Diameter | 152 mm |
| Length | 1416 mm |
| Weight | 175 kg |