|  | Add-on magnetic head pulley - 800 mm Article number: RHAM32080000 |
| --- | --- |

Attributes

| Description | Add-on magnetic head pulley - 800 mm |
| --- | --- |
| Article number | RHAM32080000 |
| Product key | RHP-P315A08-080-RSD-BA |
| Shaft type | Replaceable |
| Min. Fe-particle size that can be caught | 0,1 mm |
| Min. weakly magnetic particles that can be caught | 0,5 mm |
| Ferrous particles disposal method | Automatic - continuous |
| Magnetic field depth | 120 mm |
| Suitable for belt width | 800 mm |
| Roller width | 850 mm |
| Interface/connection | Shaft Ø50h7 (2x) (for SKF SNL 511-609) for SEW SA67 |
| Max. operating temperature | 60 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| Magnet width | 620 mm |
| Magnetic system | Permanent Neoflux™ magnet |
| Magnetic layout | Axial pole with 8 poles |
| Magnet quality | Neodymium N-38 |
| Magnet field strength (flux density) (±10%) | 6000 gauss |
| Motor drive | Not included |
| Diameter | 315 mm |
| Length | 1210 mm |