|  | Neodymium overband magnet - 1400 mm Article number: RONC140341 |
| --- | --- |

Attributes

| Description | Neodymium overband magnet - 1400 mm |
| --- | --- |
| Article number | RONC140341 |
| Product key | ROP-N4-C-140-W-G-R-B-B-M-NA |
| Suitable for cross mounting | Yes |
| Suitable for inline mounting | Yes |
| Min. Fe-particle size that can be caught | 1 mm |
| Advised max. installation height | 390 mm |
| Ferrous particles disposal method | Automatic - continuous |
| Suitable for belt width | 1400 mm |
| Installation/transport interface | Lifting eye Ø18 mm (8x) |
| Material framework | Stainless steel |
| Material covering | Stainless steel |
| Surface treatment/finishing magnet(s) | Painted |
| Surface treatment/finishing rollers | Painted |
| Surface treatment/finishing frame | Grit blasted |
| Surface treatment/finishing covers | Grit blasted |
| Colour rollers | Dark grey (Black grey): RAL7021 |
| Max. operating temperature | 60 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| Conveying length | 1910 mm |
| Belt width | 700 mm |
| Belt speed | 1,5 m/s |
| Belt type | Rubber, with cleats (EP250/2 3+1,5) |
| Belt cleat type | T50/20 |
| No. of cleats | 14 |
| Magnetic system | Permanent Neoflux™ magnet |
| Magnet quality | Neodymium N-38 |
| Magnet dimensions | 1600x550x120 mm |
| Main pole dimensions | 1400x550x120 mm |
| Motor drive | SEW electro motor with gear box |
| Motor power | 2,2 kW |
| Motor frequency | 50 Hz |
| Motor voltage | 230 (Δ) / 400 (Y) V AC |
| Motor position / operating side | Right (seen in transport direction) |
| Motor rotational speed (out) | 182 rpm |
| Motor dust/water protection class | IP55 |
| Duty cycle | 100%, 24 hours/day |
| Including central greasing point on both sides for bearings | No |
| Length | 2180 mm |
| Width | 1090 mm |
| Height | 650 mm |
| Weight | 830 kg |