|  | Valve box Article number: SPKK400007 |
| --- | --- |

Attributes

| Description | Valve box |
| --- | --- |
| Article number | SPKK400007 |
| Product key | SPKK-S-400-700-C-NA |
| Switching | Electrical/pneumatic |
| Build-in length | 700 mm |
| Product inlet size | #400 mm |
| Product outlet size | #400 mm |
| Ferrous particles outlet size | 400x320 mm |
| Interface connection type/size | Rectangular flanges with hole pattern acc. to drawing |
| Material housing | AISI304 (SS 1.4301) |
| Material valve rubber flap | Silicone VMQ, 60° Shore, EC1935/FDA, transparent |
| Surface treatment/finishing | S1: Grit blasted, Ra 3 µm |
| Welds inside the product channel | WE: Continuously welded |
| Welds outside the product channel | WE: Intermittently welded |
| Max. operating temperature | 80 °C |
| Max. operating temperature in ATEX environment | 60 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| ATEX marking | - |
| Solenoid valve electric supply | 24 V DC |
| Pneumatic supply | 6 - 8 bar |
| Length | 834 mm |
| Width | 608 mm |
| Height | 700 mm |
| Weight | 63 kg |