|  | Timing belt electromagnetic conveyor - 2000 mm Article number: TCMO100222 |
| --- | --- |

Attributes

| Description | Timing belt electromagnetic conveyor - 2000 mm |
| --- | --- |
| Article number | TCMO100222 |
| Product key | HCT-E-B100-2000-C-R-N |
| Max. sheet thickness | 5 mm |
| Interface/connection | Shaft Ø35mm for gear motor (right-hand) |
| Electrical connection | To connection box |
| Min./max. ambient temperature | -5 to 50 °C |
| Conveying length | 2000 mm |
| Belt width | 100 mm |
| Max. belt speed | 3 m/s |
| Max. acceleration | 6 m/s² |
| Belt type | BATK10-PAZ |
| Magnetic system | Electromagnet |
| Magnet quality | - |
| Magnet power | 47 W |
| Magnet voltage | 24 V DC |
| Motor drive | Not included |
| Length | 2110 mm |
| Width | 240 mm |
| Height | 110 mm |
| Weight | 101 kg |