|  | Pneumatischer Magnetgreifer - 70 mm Artikel Nummer: TPGC070078 |
| --- | --- |

Eigenschaften

| Beschreibung | Pneumatischer Magnetgreifer - 70 mm |
| --- | --- |
| Artikel Nummer | TPGC070078 |
| Produktschlüssel | HGR-SQ-070-FL-R-G-S |
| Switching | Pneumatic: compressed air or vacuum |
| Dimension lifting surface | Ø70 mm |
| Advised working load | 175 N |
| Remark Advised working load | Under ideal conditions - with safety factor 3 acc. EN13155 |
| Max. magnetic tear off force | 530 N |
| Remark max. magnetic tear off force | Under ideal conditions |
| Min. advised sheet thickness | 2 mm |
| Mounting interface | Threaded hole M6 (8x) + Threaded hole M10 |
| Pneumatic connection | G1/8" (2x) |
| Material housing | Aluminium / AISI316L (SS 1.4404) (screw cap) |
| Surface treatment/finishing | Anodized |
| Colour | Red |
| Min./max. ambient temperature | 0 to 70 °C |
| Switching vacuum pressure | -0,3 bar |
| Operating pressure | Max. 4 bar |
| Air consumption per cycle (1x up, 1x down) | 0,568 l |
| Magnetic system | Permanent Neoflux® magnet |
| Magnetic layout (number of magnet poles) | 6 |
| Magnet quality | Neodymium N-45 |
| Incl. friction pad on magnetic surface | No |
| Length | 70 mm |
| Width | 70 mm |
| Height | 70 mm |
| Weight | 0,7 kg |