|  | MagVacu® combi gripper - 42 mm Article number: TPMV040028 |
| --- | --- |

Attributes

| Description | MagVacu® combi gripper - 42 mm |
| --- | --- |
| Article number | TPMV040028 |
| Product key | HGC-RO-040-VR-R-M-F |
| Type | - |
| Switching | Pneumatic |
| Dimension lifting surface | Ø41 mm |
| Advised working load | 47 N |
| Remark Advised working load | Under ideal conditions - with safety factor acc. EN13155 |
| Max. magnetic tear off force | 35 N |
| Remark max. magnetic tear off force | Under ideal conditions |
| Max. vacuum force | 70 N |
| Remark max. vacuum force | Under ideal conditions |
| Min. advised sheet thickness | 2 mm |
| Mounting interface | Threaded hole M8 |
| Pneumatic connection | M5 (2x) |
| Material friction pad/ring | Polyurethane (PUR) 60° Shore A |
| Material housing | Aluminium |
| Surface treatment/finishing | Anodized |
| Colour | Red |
| Min./max. ambient temperature | 0 to 70 °C |
| Operating pressure | Max. 4 bar |
| Magnetic system | Permanent Neoflux® magnet - course pole |
| Magnetic layout (number of magnet poles) | 2 |
| Magnet quality | Neodymium N-45 |
| Length | 42 mm |
| Width | 42 mm |
| Height | 56 mm |
| Weight | 0,09 kg |