|  | Pneumatic sheet separator - 300 mm Article number: TPSP123301 |
| --- | --- |

Attributes

| Description | Pneumatic sheet separator - 300 mm |
| --- | --- |
| Article number | TPSP123301 |
| Product key | HSS-N-PF-140x176-0337-B |
| Max. sheet thickness | 4 mm |
| Max. stack height | 300 mm |
| Recommended working surface (for clean sheets) | 34 dm² |
| Recommended working surface (for sticky sheets) | 22 dm² |
| Mounting interface | Threaded hole M8 (4x) |
| Pneumatic connection | G1/8" (2x) |
| Material housing | AISI304 (SS 1.4301) |
| Surface treatment/finishing | - |
| Min./max. ambient temperature | -20 to 50 °C |
| Operating pressure | 5 - 10 bar |
| Magnetic system | Permanent Neoflux™ magnet |
| Magnet quality | Neodymium N-45 |
| Control | Pneumatic - Advised: 5/3-valve |
| Incl. air nozzles | - |
| Depth | 175 mm |
| Width | 140 mm |
| Height | 353 mm |
| Weight | 24 kg |