|  | Sheet separator, rod type - 100 mm Article number: TRSP341600 |
| --- | --- |

Attributes

| Description | Sheet separator, rod type - 100 mm |
| --- | --- |
| Article number | TRSP341600 |
| Product key | HSS-N-RD-D34-0160-B |
| Max. sheet thickness | 3 mm |
| Max. stack height | 100 mm |
| Recommended working surface (for clean sheets) | 5 dm² |
| Recommended working surface (for sticky sheets) | 3 dm² |
| Mounting interface | Threaded hole M10 |
| Material housing | Stainless steel |
| Surface treatment/finishing | S9: Polished, Ra 0,4 µm |
| Welds | WM: continuously welded |
| Min./max. ambient temperature | -20 to 60 °C |
| Magnetic system | Permanent Neoflux™ magnet |
| Magnet quality | Neodymium N-35 |
| Diameter | 33,6 mm |
| Height | 160 mm |
| Weight | 0,8 kg |