|  | Unidad magnética - 3xN52-Ø52,4 mm - SECA3030 Número de artículo: E0103537 |
| --- | --- |

características

| Descripción | Unidad magnética - 3xN52-Ø52,4 mm - SECA3030 |
| --- | --- |
| Número de artículo | E0103537 |
| Clave del producto | SECA-S-3030-03V-ES MAGNET INSE |
| Product name | Cleanflow - automatic discontinuouosly cleaning insert |
| Suitable for food production | For non-perishable foods only |
| Max. particle size | 10 mm |
| Min. Fe-particle size that can be caught | 0,03 mm |
| Production stop required during ferrous particles disposal | Yes |
| Cleaning (ferrous particles disposal) method | Automatic |
| Pneumatic connection | For 6 mm hose |
| Materials in contact with product | AISI304 (SS 1.4301) / AISI316L (SS 1.4404) |
| Material seal rings between housing and magnetic bars | Silicone 714177 coated, 70° Shore, EC1935/FDA, red |
| Surface finishing (interior/exterior) | S5: Polished, Ra 0.4 Âµm, excl. welds & end faces |
| Max. operating temperature | 80 °C |
| Min./max. ambient temperature | -10 to 60 °C |
| Magnetic system | Magnetic bar in tube |
| Magnet quality | Neodymium N-52 |
| Magnetic bar size | Ø52,4 mm (wall thickness 1,2 mm) |
| Number of magnetic bars | 3 |
| Flux density on magnet tube in contact with product (±10%) | 12000 gauss |
| Control | Pneumatic |
| Length | 781 mm |
| Width | 470 mm |
| Height | 111 mm |
| Weight | 38 kg |