|  | Filtro magnético industrial de polo externo - DN150 Número de artículo: E0119190 |
| --- | --- |

características

| Descripción | Filtro magnético industrial de polo externo - DN150 |
| --- | --- |
| Número de artículo | E0119190 |
| Clave del producto | SFN-150-IF-02O-N-F1m-S-B-B-B |
| Max. particle size | 50 mm |
| Min. Fe-particle size that can be caught | 1 mm |
| Production stop required during ferrous particles disposal | Yes |
| Cleaning (ferrous particles disposal) method | Manual |
| Ferrous particles disposal via | Manually by moving away the extractor plate |
| Build-in length | 360 mm |
| In-/outlet shape | Round |
| Product inlet size | Ø150 mm |
| Product outlet size | Ø150 mm |
| Interface connection type/size | DN150 acc. EN 1092-1 PN16 |
| Deflector | Fixed (welded) |
| Materials in contact with product | AISI316 (SS 1.4401) / AISI316L (SS 1.4404) |
| Material seal between housing and door/magnet unit | Silicone VMQ, 70° Shore, EC1935/FDA, red |
| Surface finishing (interior/exterior) | S1: Grit blasted, Ra 3 µm |
| Extra wear-resistant coating | No |
| Welds inside the product channel | Wm: Continuously welded |
| Welds outside the product channel | Wm: Continuously welded where possible |
| Max. operating temperature | 60 °C |
| Min./max. ambient temperature | -10 to 40 °C |
| Max. operating pressure (pipeline pressure) | 10 bar |
| Test pressure | 15 bar |
| Magnetic system | Permanent Neoflux® magnet |
| Magnet quality | Neodymium N-35 |
| Magnet field strength (flux density) (±10%) | 5000 gauss |
| Flux density on extractor wedge (±10%) | 6400 gauss |
| Depth | 428 mm |
| Width | 525 mm |
| Height | 360 mm |
| Weight | 80 kg |