|  | Medidor de gauss/Teslameter - 1D - HGM09 + toolkit Número de artículo: E0136559 |
| --- | --- |

características

| Descripción | Medidor de gauss/Teslameter - 1D - HGM09 + toolkit |
| --- | --- |
| Número de artículo | E0136559 |
| Clave del producto | MS-GM CASE ECS_OVERBAND ASSMNT |
| Suitable for | 1D measurement of static/DC or freq.-varying/AC magn. field |
| PC interface | USB 2.0 (keyboard mode, SCPI) |
| Supply voltage | 100 - 240 V AC; 50/60 Hz |
| Max. current | 0,3 A |
| Measuring range (resolution) | 10 mT - 4,5 T (1 µT - 1 mT) (see manual) |
| Battery | AA 1,2 V NiMH (2x, rechargeable, included) |
| Frequency range | 0 Hz - 5 kHz (effective value) |
| Accuracy | DC: ±0,5% (up to 1,5T), ±1% (from 1,5T); Peak: ±2%; AC: ±2% |
| Peak hold | Yes |
| Probe dimensions | 3,5 x 1,35 x 46 mm |
| Alarm function | - |
| Incl. power supply | Yes |
| Incl. USB cable | Yes |
| Incl. polarity indicator | No |
| Incl. certificate of calibration | Yes |
| Incl. rubber boot | No |
| Incl. data recording (with software available for download) | No |
| Incl. carrying case | Yes |
| Including steel objects and non-magnetic tape measure | Yes |
| Length | 450 mm |
| Width | 500 mm |
| Height | 150 mm |
| Weight | 2 kg |