|  | Paletizador magnético para grúa de pórtico - 1200x1000 mm Número de artículo: E0168606 |
| --- | --- |

características

| Descripción | Paletizador magnético para grúa de pórtico - 1200x1000 mm |
| --- | --- |
| Número de artículo | E0168606 |
| Clave del producto | HPA-N-IF-SS-1250-1050-VS3P |
| Pallet size | 1200 x 1000 mm |
| Suspension unit type | Robot arm conn. PCD200; 8xØ13,5 + 4xØ12H7, PCD160; 4xØ10H7mm |
| Pneumatic connection | For 8 mm hose |
| Electrical connection | To sensor connection box |
| Material housing | Stainless steel |
| Surface treatment/finishing | S1: Grit blasted, Ra 3 µm |
| Min./max. ambient temperature | -20 to 40 °C |
| Operating pressure | 6 - 8 bar |
| Air consumption | 8 liter/stroke (with 2 cylinders, 6 bar operating pressure) l |
| Magnetic system | Permanent Neoflux® magnet - fine pole |
| Magnet quality | Neodymium N-35 |
| Magnet dimensions | 1222 x 1022 mm |
| Power supply sensor connection box | 12-30 V DC; max. 10 W |
| Control | Electrical |
| Incl. pallet grippers | Yes |
| Incl. vacuum grippers | Yes |
| On/off sensors with terminal box | Yes |
| Depth | 1364 mm |
| Width | 1050 mm |
| Height | 464 mm |
| Weight | 315 kg |