|  | Imán overband de neodimio - 1400 mm Número de artículo: RONC140331 |
| --- | --- |

características

| Descripción | Imán overband de neodimio - 1400 mm |
| --- | --- |
| Número de artículo | RONC140331 |
| Clave del producto | ROP-N3-C-140-W-G-R-B-B-M-NA |
| Suitable for cross mounting | Yes |
| Suitable for inline mounting | Yes |
| Min. Fe-particle size that can be caught | 1 mm |
| Advised max. installation height | 310 mm |
| Ferrous particles disposal method | Automatic - continuous |
| Suitable for belt width | 1400 mm |
| Installation/transport interface | Lifting eye Ø18 mm (8x) |
| Material framework | Stainless steel |
| Material covering | Stainless steel |
| Surface treatment/finishing magnet(s) | Painted |
| Surface treatment/finishing rollers | Painted |
| Surface treatment/finishing frame | Grit blasted |
| Surface treatment/finishing covers | Grit blasted |
| Colour motor | Black (Jet Black): RAL9005 |
| Colour rollers | Dark grey (Umbra grey): RAL7022 |
| Max. operating temperature | 60 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| Conveying length | 1910 mm |
| Belt width | 600 mm |
| Belt speed | 1,5 m/s |
| Belt type | Rubber, with cleats (EP250/2 3+1,5) |
| Belt cleat type | T50/20 |
| No. of cleats | 12 |
| Magnetic system | Permanent Neoflux® magnet |
| Magnet quality | Neodymium N-38 |
| Magnet dimensions | 1600x400x80 mm |
| Main pole dimensions | 1400x400x80 mm |
| Motor drive | SEW electro motor with gear box |
| Motor power | 1,5 kW |
| Motor frequency | 50 Hz |
| Motor voltage | 230 (Δ) / 400 (Y) V AC |
| Motor position / operating side | Right (seen in transport direction) |
| Motor rotational speed (out) | 184 rpm |
| Motor dust/water protection class | IP55 |
| Duty cycle | 100%, 24 hours/day |
| Including central greasing point on both sides for bearings | Yes |
| Length | 2180 mm |
| Width | 980 mm |
| Height | 650 mm |
| Weight | 560 kg |