|  | Electroimán bala - incl. caja de válvulas Número de artículo: SPAA001063 |
| --- | --- |

características

| Descripción | Electroimán bala - incl. caja de válvulas |
| --- | --- |
| Número de artículo | SPAA001063 |
| Clave del producto | SPA-3-V-TC-FO-ST-Ex-D-B |
| Min. Fe-particle size that can be caught | 0,1 mm |
| Production stop required during ferrous particles disposal | Yes |
| Cleaning (ferrous particles disposal) method | Automatic |
| Ferrous particles disposal via | Pneumatically actuated valve box underneath product channel |
| Build-in length | 1533 mm |
| Inlet shape | Round |
| Outlet shape | Round (product) / Square (FE-parts) |
| Product inlet size | Ø250 mm |
| Product outlet size | Ø250 mm |
| Ferrous particles outlet size | 240x150 mm |
| Inlet interface connection type/size | DN250 acc. DIN 24154 R4 |
| Outlet interface connection type/size | DN250 acc. DIN 24154 R4 |
| Pneumatic connection | For 6 mm hose |
| Electrical connection | To connection box |
| Max. capacity | 100 m³/h |
| Adjustable fasteners on cleaning/inspection door | Yes |
| Materials in contact with product | Steel / AISI316 (SS 1.4401) / Nickel plated steel |
| Material seal between housing and door/magnet unit | Silicone sponge rubber VMQ, white |
| Surface treatment/finishing | Painted |
| Extra wear-resistant coating | No |
| Welds inside the product channel | WB: Welded shut |
| Welds outside the product channel | WB: Welded shut |
| Colour housing | White (Traffic white): RAL 9016 |
| Colour door | Blue (Ultramarine blue): RAL 5002 |
| Max. product flow temperature | 80 °C |
| Max. product flow temperature in ATEX environment | 40 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| ATEX marking | Ex II 1/2 D T130 °C |
| Suited for use in ATEX zone | 20, 21 and 22 inside, 21 and 22 outside |
| Dust/water protection class | IP54 |
| Magnetic system | Electromagnet |
| Magnet diameter | 250 mm |
| Angle magnet cone | 60 ° |
| Magnet field strength (flux density) (±10%) | 2000 gauss |
| Magnet voltage | 24 V DC |
| Magnet power cold | 155 W |
| Magnet power warm | 112 W |
| Pneumatic supply | 6 - 8 bar |
| Duty cycle | 100%, as long as product is flowing through (cooling effect) |
| Associated control unit (not included) | ESBP310001 |
| Remark Control | This machine has to be ordered with mentioned control unit |
| Including integrated valve box for Fe-particles disposal | Yes |
| Thermistor signals | 120 °C (warning) and 130 °C (shut off) |
| Safety device between housing and door/magnet unit | Coded magnetic switch (Steute, Ex II 2D) |
| Depth | 570 mm |
| Width | 622 mm |
| Height | 1533 mm |
| Weight | 200 kg |