|  | Cleanflow - limpieza automática continua - rotativo - #400 mm Número de artículo: SRCC400101 |
| --- | --- |

características

| Descripción | Cleanflow - limpieza automática continua - rotativo - #400 mm |
| --- | --- |
| Número de artículo | SRCC400101 |
| Clave del producto | SRCC-4040-11S-N-NA-F1H-L-B-F |
| Suitable for food production | No |
| Min. particle size | 0,2 mm |
| Max. particle size | 10 mm |
| Min. Fe-particle size that can be caught | 0,03 mm |
| Production stop required during ferrous particles disposal | No |
| Cleaning (ferrous particles disposal) method | Automatic |
| Ferrous particles disposal via | Pneumatically moving magnets, cleaning outside channel |
| Ferrous particles disposal collection unit | No |
| Build-in length | 600 mm |
| Inlet shape | Square |
| Outlet shape | Square |
| Product inlet size | #400 mm |
| Product outlet size | #400 mm |
| Ferrous particles outlet size | #120 mm |
| Interface connection type/size | Square flanges with hole pattern acc. to drawing |
| Pneumatic connection | For 6 mm hose |
| Electrical connection | To control unit |
| Max. capacity | 60 m³/h |
| Version housing | Welded |
| Materials in contact with product | AISI316 (SS 1.4401) / AISI316L (SS 1.4404) |
| Material seal ring between rotor and magnetic bars | NBR |
| Material brush between housing and rotor | Nylon (Polyamide, PA6) |
| Material gasket inspection hatch(es) | Silicone VMQ, 60° Shore, EC1935/FDA, transparent |
| Surface finishing (interior/exterior) | S1: Grit blasted, Ra 3 µm |
| Extra wear-resistant coating | No |
| Welds inside the product channel | WH: Continuously welded |
| Welds outside the product channel | WH: Welded shut |
| Max. operating temperature | 60 °C |
| Max. operating pressure (pipeline pressure) | 0 bar |
| Min./max. ambient temperature | -5 to 40 °C |
| Magnetic system | Magnetic bar in tube |
| Magnet quality | Neodymium N-42 |
| Magnetic bars position / behaviour during production | Rotating, magnetic bars placed in a rotor |
| Magnetic bar size | Ø34 mm |
| Number of magnetic bars | 11 |
| Flux density on magnet tube in contact with product (±10%) | 9400 gauss |
| Control unit power supply | 3x400 V AC + N + PE; 50 Hz |
| Pneumatic supply magnetic bars | 4 - 6 bar |
| Motor drive | SEW electro motor with gear box |
| Motor power | 0,12 kW |
| Motor current | 0,68 (Δ) / 0,39 (Y) A |
| Motor frequency | 50 Hz |
| Motor voltage | 230 (Δ) / 400 (Y) V AC |
| Motor rotational speed (out) | 5 rpm |
| Motor dust/water protection class | IP65 |
| Control | PLC Siemens LOGO! logic Module (included) |
| Incl. all bars magnet in/out sensoring | Yes |
| Length | 2200 mm |
| Width | 630 mm |
| Height | 764 mm |
| Weight | 450 kg |