|  | Transportador deslizante tipo cuello de cisne - 250 mm Número de artículo: TCBA250001 |
| --- | --- |

características

| Descripción | Transportador deslizante tipo cuello de cisne - 250 mm |
| --- | --- |
| Número de artículo | TCBA250001 |
| Clave del producto | HSC-S-250-0800-1250-60-B-B-BR |
| Capacity (for punch chips Ø7x1) | 20 kg/min |
| Material housing / slide plate | AISI304 (SS 1.4301) |
| Material underframe | S235JRC+C (St 1.0122) |
| Surface treatment/finishing | Painted (except slide plate) |
| Colour covering | Dark grey (Black grey): RAL7021 |
| Colour underframe | Red (Traffic red): RAL3020 |
| Min./max. ambient temperature | -20 to 60 °C |
| Waterproof | No |
| Infeed height | 142 mm |
| Outfeed height | 1200 mm |
| Conveying length | 2250 mm |
| Conveying width | 325 mm |
| Magnet width | 250 mm |
| Conveying speed | 0,3 m/s |
| Magnetic system | Permanent Neoflux® magnet |
| Motor drive | SEW electro motor with gear box |
| Motor power | 0,55 kW |
| Motor frequency | 50 Hz |
| Motor voltage | 230 (Δ) / 400 (Y) V AC |
| Motor position / operating side | Right (seen in transport direction) |
| Motor rotational speed (out) | 29 rpm |
| Motor dust/water protection class | IP54 |
| Angle | 60 ° |
| Length | 2252 mm |
| Width | 526 mm |
| Height | 1540 mm |