|  | Pinza magnética neumática - 24 mm Número de artículo: TPGC024078 |
| --- | --- |

características

| Descripción | Pinza magnética neumática - 24 mm |
| --- | --- |
| Número de artículo | TPGC024078 |
| Clave del producto | HGR-SQ-024-FL-R-M-F |
| Switching | Pneumatic: compressed air or vacuum |
| Dimension lifting surface | Ø24 mm |
| Advised working load | 11 N |
| Remark Advised working load | Under ideal conditions - with safety factor 3 acc. EN13155 |
| Max. magnetic tear off force | 34 N |
| Remark max. magnetic tear off force | Under ideal conditions |
| Min. advised sheet thickness | 1 mm |
| Mounting interface | Threaded hole M5 (4x) |
| Pneumatic connection | M5 (2x) |
| Material housing | Aluminium / AISI316L (SS 1.4404) (screw cap) |
| Surface treatment/finishing | Anodized |
| Colour | Red |
| Min./max. ambient temperature | 0 to 70 °C |
| Switching vacuum pressure | -0,3 bar |
| Operating pressure | Max.: 6, recommended: 4 bar |
| Air consumption per cycle (1x up, 1x down) | 0,052 l |
| Magnetic system | Permanent Neoflux™ magnet |
| Magnetic layout (number of magnet poles) | 2 |
| Magnet quality | Neodymium N-52 |
| Incl. friction pad on magnetic surface | No |
| Length | 24 mm |
| Width | 24 mm |
| Height | 63 mm |
| Weight | 0,1 kg |