|  | Gaussmètre / teslamètre - 3D - VGM Nº d'article: 65.0000067 |
| --- | --- |

Caractéristiques techniques

| La description | Gaussmètre / teslamètre - 3D - VGM |
| --- | --- |
| Nº d'article | 65.0000067 |
| Clé de produit | MS-GM GAUSSMETER 3 AXIS VGM |
| Suitable for | 3Dmeasurement (XYZ) of static/DC magn.field (magnitude+dir.) |
| PC interface | USB 2.0 (data logging) |
| Supply voltage | 100 - 240 V AC; 50/60 Hz |
| Max. current | 1 A |
| Measuring range (resolution) | ± 79,999 mT (1 µT) |
| Battery | 9V Alkaline (included) |
| Frequency range | - (static only) |
| Accuracy | 1% of reading (16 to 29°C), 2% of reading (-4 to 65°C) |
| Peak hold | Yes (X,Y, Z + magnitude) |
| Probe dimensions | 6,3 x 6,3 x 50 mm (non-detachable) |
| Alarm function | Adjustable (range 0,1 - 79 mT) |
| Incl. power supply | Yes |
| Incl. USB cable | Yes |
| Incl. polarity indicator | No |
| Incl. certificate of calibration | Yes |
| Incl. rubber boot | Yes |
| Incl. data recording (with software available for download) | Yes |
| Incl. carrying case | Yes |
| Including steel objects and non-magnetic tape measure | No |
| Length | 147 mm |
| Width | 97 mm |
| Height | 51 mm |
| Weight | 0,33 kg |