|  | Système de rouleau d'entraînement à haut gradient - 600 mm Nº d'article: E0152603 |
| --- | --- |

Caractéristiques techniques

| La description | Système de rouleau d'entraînement à haut gradient - 600 mm |
| --- | --- |
| Nº d'article | E0152603 |
| Clé de produit | RHPS-06-H153-RR-MS-SM-1PU16-N |
| Max. particle size | 40 mm |
| Min. Fe-particle size that can be caught | 0,03 mm |
| Min. weakly magnetic particles that can be caught | 0,1 mm |
| Ferrous particles disposal method | Automatic - continuous |
| Length vibratory feeder | 766 mm |
| Angle vibratory feeder | 2 ° |
| Vibratory feeder bin wear protection | None (Basic stainless steel) |
| Separation flap type | Standard (coarse) |
| Working width | 600 mm |
| Material framework | Steel |
| Material covering | Stainless steel |
| Surface treatment/finishing covers | Grit blasted |
| Surface treatment/finishing frame (P1) | S6: Powder coated |
| Colour frame | Dark grey (Black grey): RAL7021 |
| Max. operating temperature | 60 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| Infeed height | 1135 (-100 / +150) mm |
| High gradient belt height (-100 / +150) (infeed/outfeed) | 1023 mm |
| Max. belt speed | 1,4 m/s |
| Belt type | 1,3 mm PU antistatic, blue, with 1 cleat (A-shaped) |
| Magnetic system | Permanent Neoflux® magnet |
| Magnetic layout | Radial pole |
| Magnet quality | Neodymium (NdFeB) |
| Flux density on belt (±10%) | 9000 gauss |
| Control unit power supply | 230 V AC + N + PE; 50 Hz |
| Motor drive feeder | Invicon vibration motor |
| Motor drive belt | Van der Graaf drum motor |
| Motor power feeder | 0,2 kW |
| Motor frequency feeder | 50 Hz |
| Motor voltage feeder | 230 (Δ) / 400 (Y) V AC |
| Motor power belt | 0,37 kW |
| Motor frequency belt | 50 Hz |
| Motor voltage belt | 230 (Δ) / 400 (Y) V AC |
| Belt motor dust/water protection class | IP66 |
| Feeder motor dust/water protection class | IP66 |
| Associated control unit (not included) | E0122431 |
| Incl. stainless steel vibratory feeder | Yes |
| Incl. stainless steel discharge chute for metals | Yes |
| Incl. height adjustable frame | Yes |
| Incl. belt change system | Yes |
| Length | 1982 mm |
| Width | 1198 mm |
| Height | 1363 mm |
| Weight | 510 kg |