|  | Unité magnétique - 5xN45SH-Ø30 mm - SECR2020 Nº d'article: E0153205 |
| --- | --- |

Caractéristiques techniques

| La description | Unité magnétique - 5xN45SH-Ø30 mm - SECR2020 |
| --- | --- |
| Nº d'article | E0153205 |
| Clé de produit | SECR-S-2020-05C-HT-MAG INSERT |
| Type | Magnet Insert for Rotating cleanflow SECR-2020 |
| Cleaning (ferrous particles disposal) method | Manual |
| Version housing | Welded |
| Materials in contact with product | AISI316 (SS 1.4401) / AISI316L (SS 1.4404) |
| Surface finishing (interior/exterior) | S5: Vibratory polished, Ra 0,4 µm, ex. welds and end faces |
| Max. operating temperature | 100 °C |
| Max. operating temperature in ATEX environment | 40 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| Magnetic system | Magnetic bar in extractor tube |
| Magnet quality | Neodymium N-45SH |
| Magnetic bars position / behaviour during production | Rotating, magnetic bars placed in a rotor |
| Magnetic bar size | Extractor tube Ø32 mm; magnetic bar Ø30 mm |
| Number of magnetic bars | 5 |
| Flux density on (internal) magnetic bar (±10%) | 13000 gauss |
| Length | 195 mm |
| Width | 160 mm |
| Height | 160 mm |
| Weight | 6 kg |