|  | Cleanflow statique à nettoyage automatique continu - #400 mm Nº d'article: SECC400201 |
| --- | --- |

Caractéristiques techniques

| La description | Cleanflow statique à nettoyage automatique continu - #400 mm |
| --- | --- |
| Nº d'article | SECC400201 |
| Clé de produit | SECC-4040-09S-N-Ex-F1E-L-B-B |
| Suitable for food production | For non-perishable foods only |
| Min. particle size | 0,2 mm |
| Max. particle size | 10 mm |
| Min. Fe-particle size that can be caught | 0,03 mm |
| Production stop required during ferrous particles disposal | No |
| Cleaning (ferrous particles disposal) method | Automatic |
| Ferrous particles disposal via | Pneumatically moving to and cleaning magnets in side channel |
| Ferrous particles disposal collection unit | No |
| Build-in length | 250 mm |
| Inlet shape | Square |
| Outlet shape | Square |
| Product inlet size | #400 mm |
| Product outlet size | #400 mm |
| Ferrous particles outlet size | #120 mm |
| Interface connection type/size | Rectangular flanges with hole pattern acc. to drawing |
| Pneumatic connection | For 6 mm hose |
| Electrical connection | To connection box |
| Max. capacity | 90 m³/h |
| Materials in contact with product | AISI304 (SS 1.4301) / AISI316 (SS 1.4401) |
| Material slide ring housing / magnet bar | PA6G (Polyamide), EC1935/FDA, natural |
| Surface finishing (interior/exterior) | S1: Grit blasted, Ra 3 µm |
| Extra wear-resistant coating | No |
| Welds inside the product channel | WE: Continuously welded |
| Welds outside the product channel | WE: Intermittently welded |
| Max. operating temperature | 60 °C |
| Max. operating temperature in ATEX environment | 40 °C |
| Min./max. ambient temperature | -5 to 40 °C |
| Max. operating pressure (pipeline pressure) | 0 bar |
| ATEX marking | Ex II 1/2 D h T130 Da/Db |
| Suited for use in ATEX zone | 20, 21 and 22 inside, 21 and 22 outside |
| Magnetic system | Magnetic bar in tube |
| Magnet quality | Neodymium N-42 |
| Magnetic bars position / behaviour during production | Static, fixed positon in 2 rows, placed in cascade |
| Magnetic bar size | Ø34 mm |
| Number of magnetic bars | 9 (4+5) |
| Flux density on magnet tube in contact with product (±10%) | 9400 gauss |
| Control unit power supply | 230 V AC; 50/60 Hz |
| Pneumatic supply magnetic bars | 4 - 6 bar |
| Control | PLC Siemens LOGO! logic Module (included) |
| Length | 1670 mm |
| Width | 626 mm |
| Height | 490 mm |
| Weight | 180 kg |