|  | Aimant tubulaire électromagnétique - 350 m³/h Nº d'article: SPEB000066 |
| --- | --- |

Caractéristiques techniques

| La description | Aimant tubulaire électromagnétique - 350 m³/h |
| --- | --- |
| Nº d'article | SPEB000066 |
| Clé de produit | SPA-6-B-FI-FO-ST-NA-D-B |
| Min. Fe-particle size that can be caught | 0,1 mm |
| Production stop required during ferrous particles disposal | Yes |
| Cleaning (ferrous particles disposal) method | Automatic |
| Ferrous particles disposal via | Pneumatically actuated valve box underneath product channel |
| Build-in length | 1200 mm |
| Inlet shape | Round |
| Outlet shape | Round |
| Product inlet size | Ø500 mm |
| Product outlet size | Ø500 mm |
| Ferrous particles outlet size | - mm |
| Inlet interface connection type/size | DN500 acc. DIN 24154 R4 |
| Outlet interface connection type/size | DN500 acc. DIN 24154 R4 |
| Electrical connection | To connection box |
| Max. capacity | 350 m³/h |
| Adjustable fasteners on cleaning/inspection door | Yes |
| Materials in contact with product | Steel / AISI316 (SS 1.4401) / Nickel plated steel |
| Material seal between housing and door/magnet unit | Silicone sponge rubber VMQ, white |
| Surface treatment/finishing | Painted |
| Extra wear-resistant coating | No |
| Welds inside the product channel | WB: Welded shut |
| Welds outside the product channel | WB: Welded shut |
| Colour housing | White (Traffic white): RAL 9016 |
| Colour door | Blue (Ultramarine blue): RAL 5002 |
| Max. product flow temperature | 80 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| ATEX marking | - |
| Dust/water protection class | IP54 |
| Magnetic system | Electromagnet |
| Magnet diameter | 500 mm |
| Angle magnet cone | 70 ° |
| Magnet field strength (flux density) (±10%) | 2400 gauss |
| Magnet voltage | 44 V DC |
| Magnet power cold | 560 W |
| Magnet power warm | 410 W |
| Pneumatic supply | 6 - 8 bar |
| Duty cycle | 100%, as long as product is flowing through (cooling effect) |
| Associated control unit (not included) | ESBP210004 |
| Remark Control | This machine has to be ordered with mentioned control unit |
| Including integrated valve box for Fe-particles disposal | No |
| Thermistor signals | 110 °C (warning) and 120 °C (shut off) |
| Safety device between housing and door/magnet unit | Coded magn switch (Telemecanique XCSDMC59010EX) (Ex II2D) |
| Depth | 920 mm |
| Width | 1105 mm |
| Height | 1200 mm |
| Weight | 520 kg |