|  | Boîte à rabat Numer artykułu: SPKK240006 |
| --- | --- |

Właściwości

| Opis | Boîte à rabat |
| --- | --- |
| Numer artykułu | SPKK240006 |
| Klucz produktu | SPKK-S-240-500-C-Ex |
| Switching | Electrical/pneumatic |
| Build-in length | 500 mm |
| Product inlet size | #240 mm |
| Product outlet size | #240 mm |
| Ferrous particles outlet size | 240x150 mm |
| Interface connection type/size | Rectangular flanges with hole pattern acc. to drawing |
| Material housing | AISI304 (SS 1.4301) |
| Material valve rubber flap | Silicone VMQ, 60° Shore, EC1935/FDA, transparent |
| Surface treatment/finishing | S1: Grit blasted, Ra 3 µm |
| Welds inside the product channel | WE: Continuously welded |
| Welds outside the product channel | WE: Intermittently welded |
| Max. operating temperature | 80 °C |
| Max. operating temperature in ATEX environment | 60 °C |
| Min./max. ambient temperature | -5 to 40 °C |
| ATEX marking | Ex II 1/2 D h T130 Da/Db |
| Suited for use in ATEX zone | 20, 21 and 22 inside, 21 and 22 outside |
| Solenoid valve electric supply | 24 V DC |
| Pneumatic supply | 6 - 8 bar |
| Length | 661 mm |
| Width | 448 mm |
| Height | 500 mm |
| Weight | 40 kg |