|  | Bloc électromagnétique suspendu - 1000 mm Nº d'article: SZBE052001 |
| --- | --- |

Caractéristiques techniques

| La description | Bloc électromagnétique suspendu - 1000 mm |
| --- | --- |
| Nº d'article | SZBE052001 |
| Clé de produit | RBE-EP8-090x092-100-A-NA |
| Min. Fe-particle size that can be caught | 3 mm |
| Advised max. installation height | 225-360 mm |
| Production stop required during ferrous particles disposal | Yes |
| Cleaning (ferrous particles disposal) method | Automatic |
| Ferrous particles disposal via | By switching off electromagnet |
| Suitable for belt width | 1000 mm |
| Installation/transport interface | Lifting eye Ø40 mm (4x) |
| Material housing | Steel |
| Surface treatment/finishing | Painted |
| Colour housing | Blue (Ultramarine blue): RAL 5002 |
| Max. operating temperature | 80 °C |
| Min./max. ambient temperature | -10 to 40 °C |
| Dust/water protection class | IP54 |
| Magnetic system | Electromagnet with air cooling |
| Magnet power | 5500 W |
| Magnet current | 50 A |
| Magnet voltage | 110 V DC |
| Duty cycle | 100%, 24 hours/day |
| Associated control unit (not included) | ESBB907001 |
| Length | 1098 mm |
| Width | 950 mm |
| Height | 468 mm |
| Weight | 1800 kg |