|  | Convoyeur clos magnétique plat - 500 mm Nº d'article: TCBC500001 |
| --- | --- |

Caractéristiques techniques

| La description | Convoyeur clos magnétique plat - 500 mm |
| --- | --- |
| Nº d'article | TCBC500001 |
| Clé de produit | HSC-F-500-1500-0000-00-B-B-BR |
| Capacity (for punch chips Ø7x1) | 42 kg/min |
| Material housing / slide plate | AISI304 (SS 1.4301) |
| Surface treatment/finishing | Painted (except slide plate) |
| Colour covering | Dark grey (Black grey): RAL7021 |
| Colour underframe | (optional): Red (Traffic red): RAL3020 |
| Min./max. ambient temperature | -20 to 60 °C |
| Waterproof | No |
| Infeed height | 294 mm |
| Outfeed height | 137 mm |
| Conveying length | 1500 mm |
| Conveying width | 575 mm |
| Magnet width | 500 mm |
| Conveying speed | 0,3 m/s |
| Magnetic system | Permanent Neoflux® magnet |
| Motor drive | SEW electro motor with gear box |
| Motor power | 0,55 kW |
| Motor frequency | 50 Hz |
| Motor voltage | 230 (Δ) / 400 (Y) V AC |
| Motor position / operating side | Left (seen in transport direction) |
| Motor rotational speed (out) | 29 rpm |
| Motor dust/water protection class | IP54 |
| Length | 1588 mm |
| Width | 776 mm |
| Height | 340 mm |