|  | EddyXpert™ - Eddy current non-ferro scheider - 1500 mm Artikelnummer: E0136511 |
| --- | --- |

Kenmerken

| Omschrijving | EddyXpert™ - Eddy current non-ferro scheider - 1500 mm |
| --- | --- |
| Artikelnummer | E0136511 |
| Product key | REC-15-E22HI-N-MAB-SM-PU-R |
| Incl. magnetic drum separator | No |
| Incl. magnetic head pulley separator | No |
| Incl. magnetic overband separator | No |
| Feeder module | Vibratory feeder |
| Max. particle size | 300 mm |
| Separates non-magnetic particles | 5-60+ mm |
| Length vibratory feeder | 2000 mm |
| Angle vibratory feeder | 5 ° |
| Vibratory feeder bin wear protection | None (Basic Mn steel bin) |
| Separation box | Medium |
| Separation flap type separation box | Standard (coarse) |
| Separation box flap operation | Manual by lever |
| Working width | 1500 mm |
| Rotational speed eddy current magnet roller | 3800 rpm |
| Material framework | Steel |
| Material covering | Steel |
| Material slide plates | Stainless steel |
| Surface treatment/finishing | S6: Powder coated (C4) |
| Colour P1: frame + covering | Dark grey (Black grey): RAL7021 |
| Colour P2: side panels | Blue (Ultramarine blue): RAL 5002 |
| Colour P2: vibratory feeder | Blue (Ultramarine blue): RAL 5002 |
| Max. operating temperature | 60 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| Infeed height | 1535 mm |
| Eddy current belt height | 1032 mm |
| Eddy current belt speed | 0 - 2,5 m/s |
| Belt type eddy current separator | 3 mm Polyurethane, green, 94° Shore |
| Belt scraper eddy current separator | Rubber (NBR) |
| Eddy current magnet roller type | 22 poles High Intensity, eccentric Ø300/Ø500 magnet rotor |
| Magnet quality | Neodymium (NdFeB) |
| Flux density eddy current magnet roller, on belt (±10%) | 3000 gauss |
| Control unit power supply | 3x400 V AC + PE; 50 Hz |
| Motor drive feeder | Koolen vibration motor |
| Motor drive eddy current belt | Van der Graaf drum motor |
| Motor drive eddy current magnet roller | AEG electro motor |
| Motor power feeder | 1,2 kW |
| Motor current feeder | 2,2 A |
| Motor frequency feeder | 50 Hz |
| Motor voltage feeder | 230 (Δ) / 400 (Y) V AC |
| Motor power eddy current belt | 2,2 kW |
| Motor current eddy current belt | 4,7 A |
| Motor frequency eddy current belt | 50 Hz |
| Motor voltage eddy current belt | 230 (Δ) / 400 (Y) V AC |
| Motor power eddy current magnet roller | 7,5 kW |
| Motor frequency eddy current magnet roller | 50 Hz |
| Motor voltage eddy current magnet roller | 400 V AC (Δ) / 690 V AC (Y) V AC |
| Motor position / operating side | Right (seen in transport direction) |
| Belt motor dust/water protection class | IP66 |
| Feeder motor dust/water protection class | IP66 |
| Eddy current magnet roller motor dust/water protection class | IP55 |
| Associated control unit (not included) | E0130127/E0130271/E0131082/E0131083/E0131084 |
| Including central greasing point on both sides for bearings | No |
| Electrical connector plate | No (direct connections to motor drives) |
| Thermal protection eddy current roller | No |
| Length | 5283 mm |
| Width | 2648 mm |
| Height | 1535 mm |
| Weight | 3260 kg |